xoExam™ UI/UX — Engineering Handoff Specification

v0.2.8 · June 9, 2026 · Hardware answers confirmed (Q3/Q7/Q8) + Smart-Cylinder/binocular toggle fix

Method Marketing Agency for Xenon Ophthalmics Inc.

0. What changed in v0.2.8 (June 9, 2026)

1. Hardware answers confirmed (Steve, Q3/Q7/Q8). The headset-sensor questions came back: pupil detection in dim vs. bright light; wavefront imaging at 4 mm (6 mm treated as confirmed per CD direction); Zernike to 10th order / 66 modes; vertex 25–30 mm; sphere-only range −14 to +14 D; cylinder range 0 to −5 D (opto-electrical spec sheet). Wavefront Refraction now presents all of these as confirmed — WFR_REEHANA_CONFIRMED = true removes every Provisional pill and pending caption, and the real sphere/cylinder ranges replace the prior Q8 placeholders. Values stay isolated in named constants for any later revision.

2. Toggle-switch rendering fix. The Smart-Cylinder auto-bracketing and binocular-fogging toggles were being forced to squares by the global 44 px touch-target rule; both are now constrained to a proper rectangle (minWidth:38, minHeight:22).

3. Doc set reissued at v0.2.8. This document and the full client/handoff doc set — Engineering Handoff Specification, Development Brief, Clinical Standards Reference, Clinical Evaluation Brief, CLAUDE.md, README.md, the Wavefront Refraction spec, and the _dist_v0.2.8/ package — are brought to v0.2.8 together.

1. What changed in v0.2.7 — accurate change log

This reissue folds the six competitive-parity enhancements for Wavefront Refraction into the single production component and eliminates the parallel v2 draft file. It supersedes v0.2.6. All sixteen sections of the prior spec remain authoritative unless updated below.

| Area | v0.2.6 | v0.2.7 |
| --- | --- | --- |
| Tests at clinical fidelity | 7 of 19 | 7 of 19 (no new test; Wavefront Refraction deepened) |
| Wavefront Refraction | base two-stage flow | + six competitive-parity enhancements merged in |
| WFR source files | 1 production + 1 v2 draft fork | 1 file only (v2 fork + harness deleted) |
| WFR report layout | single scrolling report | tabbed report (Summary · Rx comparison · Day & night · Vision simulation · Wavefront · Progression) |
| Newest engineering integration item | Certify & close as Rx-release system event | Steve's hardware answers (Q3/Q7/Q8) confirmed — 6 mm / vertex / Rx-range values locked in |

Files touched in v0.2.7

| File | Change |
| --- | --- |
| components/WavefrontRefractionTest.jsx | Replaced — the six-enhancement build folded into the single production file (~2280 lines, all WFR_-prefixed); component name kept as WavefrontRefractionTest. All hardware values isolated in named constants at the top (WFR_ANALYSIS_DIA, WFR_SIXMM_PROVISIONAL, WFR_ZERNIKE_MAX_ORDER/WFR_ZERNIKE_MODES, WFR_VERTEX_MM, WFR_SPHERE_RANGE/WFR_CYL_RANGE) so unconfirmed items flip with a one-line edit. |
| components/WavefrontRefractionTest v2.jsx | Deleted — the parallel draft fork. One file, one source of truth (CD direction: a divergent second copy is a versioning hazard). |
| Wavefront Refraction v2 Demo.html · wfr-v2-demo/ | Deleted — the fork's standalone demo + single-file build. |
| index.html | Edited — title bumped to v0.2.7. No routing change: the shell already imported WavefrontRefractionTest.jsx and rendered <WavefrontRefractionTest/>, so the enhanced flow is now live in the full app. |
| briefs/WavefrontRefraction_Clinical_Spec_v2.md | Added — supersedes the v1 spec; documents the six enhancements and the doctor-led posture. |
| deploy.html · deploy-new-site.html | Edited — version strings bumped to v0.2.7. |
| CLAUDE.md · README.md | Updated — v0.2.7 version-log entry, WFR section rewritten (merge), component index, test-stream table, status, dist note. |

The six enhancements (all doctor-led-safe — measurements/simulations, never verdicts)

1. PSF + simulated-VA before/after — point-spread-function render + simulated acuity, habitual-vs-new Rx, driven by the Zernike set (Simulation tab).

2. Binocular balance step — after both eyes, via fogging / alternate occlusion (no prism hardware needed); subjective subjStep:'binocular'.

3. Multi-source Rx comparison — objective / subjective / habitual / unaided columns with spherical-equivalent deltas; habitual entered manually now, auto-pulled from history later (Rx-comparison tab).

4. Photopic-vs-mesopic day-&-night — 4 mm column + 6 mm column (both hardware-confirmed) with a night-shift Diff row (Day-&-night tab).

5. Smart-Cylinder auto-bracketing — JCC step ladder auto-sized to |cyl|, narrows one rung per reversal; toggle.

6. Refraction progression tracker — spherical-equivalent trend + D/yr rate vs age-banded reference; the at-risk call is left to the clinician (Progression tab).

Hardware values confirmed (Steve, Q3/Q7/Q8). The 6 mm pupil columns, vertex distance (25–30 mm), and sphere/cylinder ranges (sphere −14 to +14 D, cylinder 0 to −5 D) are confirmed and isolated in named constants. Any later hardware revision is a one-line edit to WFR_SIXMM_PROVISIONAL — no rebuild, no separate file.

Framing. The program remains pre-beta, active development pending final feedback from MPR and Xenon's Chief Medical Officer + doctor panel. v0.2.7 is the integration version, not a finished milestone.

2. Engineering quick start (unchanged from v0.2.1)

•  Stack: React 18.3.1 + Babel Standalone 7.29.0 via CDN. No build toolchain. index.html is the single entry point.

•  Open index.html directly in a modern browser (Chrome / Edge recommended). The prototype renders immediately — no server, no install, no build.

•  Production target: React Native + Kotlin Android tablet. The .jsx source is structured for mechanical port — inline styles, hooks-only, no exotic dependencies. See the v0.2.1 spec §10 for the full RN portability table.

3. ExamShell Component Interface Contract (unchanged)

Every test component must conform to this contract exactly. Wavefront Refraction is the seventh test built against it.

js

function [TestName]({ onBack, tweaks }) {

const accent = tweaks?.accentColor || '#1f8eff';

// ...

return (

<ExamShell

title="[Test name — sentence case]"

accent={accent}

onBack={onBack}

patientName={patientName}

patientId={patientId}

phase={phase}

elapsed={elapsed}

onBegin={startTest}

onFinish={() => setPhase('report')}

onNewTest={resetTest}

rightPanel={phase === 'testing' ? <RightSidebar /> : null}

>

{phase === 'ready'   && renderReady()}

{phase === 'testing' && renderTesting()}

{phase === 'report'  && renderReport()}

</ExamShell>

);

}

Object.assign(window, { [TestName] });

ExamShell owns the canonical Cancel UX: back arrow hidden during testing, labeled red "Cancel test" button takes over, opens confirm modal. Test components MUST NOT build their own cancel dialogs.

For tests with sub-stage state machines inside the testing phase (EOM, Pupillometry, and now Wavefront Refraction's objective→subjective sequence), finishAvailable() is the local helper that gates ExamShell's green "Finish & Report" button — only at the last enabled sub-step's completion state.

4. Wavefront Refraction — engineering-specific integration touchpoints

The clinical spec (briefs/WavefrontRefraction_Clinical_Spec_v1.md) is the authoritative source. Engineering touchpoints specific to this test:

4.1 Certify & close is a SYSTEM EVENT — the most consequential integration point in the suite

Wavefront Refraction produces the verified prescription (Rx) — the combined output of the objective wavefront capture and the subjective phoropter refinement. Its "Certify & close" action is not a UI-only state change. It is the certification event that releases the Rx to the downstream xoFit fitting/finishing job object.

•  The prototype surfaces this as a button with a "Doctor sign-off · releases Rx to job object" sub-label.

•  MPR must wire Certify & close to emit the Rx-release event to the job/order pipeline. Until certified, the Rx is not released downstream.

•  This couples to the role/permissions model (kickoff agenda item 5): only an authorized doctor identity may certify.

4.2 Two-stage capture model

| Stage | UI phase | Behavior | Hardware contract |
| --- | --- | --- | --- |
| Objective | ExamShell testing (sub-stage objective) | Monocular wavefront capture, OD then OS. Live eye-scan look — dark viewer, breathing pupil, accent sweep line, rotating centering ring, red alignment dot that turns the backdrop green on lock. Sub-stages init → calibrating → capturing → complete. | Liquid-lens / eye-tracking capability drives readiness panel values and green-lock timing — engineer confirmation needed. Per-eye scan count (1–3) documents reliability; the wavefront retries on movement/blink and averages. |
| Subjective | ExamShell testing (sub-stage subjective) | Liquid-lens digital phoropter. Step sequence per eye setup → sphere (MPMVA) → jcc-axis → jcc-power → [mpmva2*] → add. | Liquid-lens power presentation per step; optotype rendering on the patient chart. |

mpmva2 (second MPMVA) auto-triggers only if cyl changed ≥ 0.50 D or axis changed ≥ 10° from objective.

4.3 Clinical non-negotiables enforced in the UI (preserve on port)

1. Axis before power (JCC). Enforced by step order — jcc-axis always precedes jcc-power. The UI cannot reach power refinement before axis is confirmed.

2. Fogging control. After objective capture a fog prompt offers +0.75 D with clinical explanation; starting MPMVA sphere = objective sphere + fog. Doctor may skip.

3. Sphere compensation note visible during cylinder power: "For each −0.50 D CYL added, add +0.25 D SPH to maintain the circle of least confusion."

4. Rx format discipline. SPH/CYL always signed + 2 dp; AXIS integer + °; ADD signed with D or "—". Helpers: WFR_fmtSph / WFR_fmtCyl / WFR_fmtAxis / WFR_fmtAdd.

5. Monocular objective. No OU averaging. Sequence OD → OS automatically via breadcrumb + transition modal.

6. No clinical claims in the report. Deliberate divergence from the other clinical-fidelity tests: NO Patient Classification banner, NO interpretive language. A refraction is a measurement-and-verification workflow, not a screening. The Clinical Summary states data only (scan counts, completion, "Final Rx pending clinician certification").

4.4 Data persistence schema additions

Wavefront Refraction adds this per-test results object (see v0.2.1 §6 for the cross-test schema):

js

{

testId: 'wavefront-refraction',

mode: 'full',                  // 'full' (objective→subjective) | 'subjective-only'

eyesTested: ['OD', 'OS'],

objective: {

OD: { sph:-1.25, cyl:-0.50, axis:165, pupilSize:6.2, analysisDia:6.0, scans:2,

rmsHOA:0.261, comaRMS:0.163, sphAb:0.087 },

OS: { /* same shape */ }

},

fogApplied: 0.75,              // D, or 0 if skipped

subjective: {

OD: { sph:-1.00, cyl:-0.50, axis:165, add:0, bcva:'20/20', jccPower:0.50 },

OS: { /* same shape */ }

},

finalRx: {

OD: { sph:-1.00, cyl:-0.50, axis:165, add:0 },

OS: { /* same shape */ }

},

notes: 'Free-text session notes',

certifiedBy: '<doctor-id>',    // populated at Certify & close — Rx-release event

certifiedAt: '<ISO timestamp>'

}

4.5 Pure-function port targets

•  WFR_fmtSph / WFR_fmtCyl / WFR_fmtAxis / WFR_fmtAdd — Rx format discipline

•  WFR_lineIsPassed(correct, total) — VA pass/fail threshold (see open item 4.6.1)

•  WFR_getBestVA(...) — additive VA notation (e.g. 20/25 +4)

•  WFR_Optotype — geometric Tumbling E / Landolt C renderer (shared with Visual Acuity)

4.6 Open items / TBD for the engineering kickoff

1. VA pass/fail threshold (WFR_lineIsPassed) currently "more than half correct." Gary to validate against a published standard before lock.

2. Certify & close → system event wiring (§4.1) — MPR to implement as the Rx-release trigger to the xoFit job object.

3. Patient-facing headset view is out of scope. This component is the tablet (technician/doctor) view only. The patient-facing headset/clicker experience (objective self-administered mode) is separate scope to define with MPR.

4. Objective readiness panel values + green-lock timing depend on liquid-lens / eye-tracking capability (engineer confirmation).

5. Open decisions — engineering kickoff agenda (cumulative, v0.2.7)

The cumulative list from v0.2.5 stands. New items added by Wavefront Refraction are flagged [R]:

1. Hardware-to-UI event contract (cross-test) — what events the headset fires, what payload, what the UI subscribes to

2. Per-test data persistence schema — final shape across all clinical-fidelity tests

3. Authentication model — login, session, doctor identity

4. Export mechanism — PDF generation, EHR forwarding, cloud sync

5. Role-based permissions — Doctor sign-off as a gate, not just a label

6. Patient identity confirmation step — currently assumed pre-test

7. Compliance scope — HIPAA, MDR class, FDA 510(k) path

8–14. Pupillometry firmware/clinical questions (v0.2.5 §4.1 — items 8–14)

15. [R] Certify & close → Rx-release system event wiring to the xoFit job object (§4.1)

16. [R] VA pass/fail threshold validation for the subjective stage (§4.6.1)

17. [R] Patient-facing headset/clicker view scope for objective self-administered mode (§4.6.3)

18. [R] Liquid-lens / eye-tracking capability for objective readiness + green-lock timing (§4.6.4)

6. Regulatory acknowledgment (unchanged)

The xoExam UI/UX prototype is not a medical device and has not been cleared by FDA, MDR, or any other regulatory body. The clinical-accuracy work in briefs/ and in the Clinical Standards Reference is a transparency document — a description of what the design adheres to — not a clinical validation report or regulatory submission. Production clinical validation is an engineering and regulatory responsibility outside Method's scope.

Note specific to Wavefront Refraction: the report deliberately makes no clinical claims — no determinations, assessments, or recommendations. This is by design: a refraction is a measurement-and-verification workflow, not a screening. The verified Rx is a measured output for clinician certification, not a diagnostic finding.

The same calibration assumption applies as for all tests: the prototype assumes the production headset delivers calibrated stimulus presentation, accurate liquid-lens power, valid wavefront measurement, and consistent conditions. Whether it does is a hardware question outside the UI/UX layer.

7. Per-test specs (current)

The product version is unified. The entire xoExam UI/UX ships as one build — currently v0.2.7. The right-hand column records *when each test last reached clinical fidelity* (the release it was last meaningfully updated in), not a separate version number for that test. A test showing an earlier release is correct and unchanged since then, not behind.

| # | Test | Spec file in `briefs/` | Reached clinical fidelity in |
| --- | --- | --- | --- |
| 1 | Visual Acuity | VisualAcuity_Clinical_Spec_v2.md | v0.2.3 (Tumbling E/C added v0.2.6) |
| 2 | Color Vision | (in CLAUDE.md + Clinical Standards Reference §2) | v0.1.7 |
| 3 | Visual Fields | VisualField_Clinical_Spec_v2.md | v0.1.8 |
| 4 | Wavefront Aberrometry | WavefrontAberrometry_Clinical_Spec_v2.md | v0.1.9 |
| 5 | Extraocular Motility | ExtraocularMotility_Clinical_Spec_v2.md | v0.2.1 |
| 6 | Pupillometry | Pupillometry_Clinical_Spec_v2.md | v0.2.5 |
| 7 | Wavefront Refraction | WavefrontRefraction_Clinical_Spec_v2.md | v0.2.6 (six enhancements merged v0.2.7) |

The remaining 12 tests are visual-fidelity placeholders. Specs are written as each one is brought to clinical fidelity.

8. Document set issued at v0.2.8

| Document | Status |
| --- | --- |
| xoExam UI-UX Engineering Handoff Specification v0.2.7.docx | This document — supersedes v0.2.6 |
| xoExam Development Brief 06-08-2026 v0.2.7.docx | New per-release brief |
| xoExam Development Brief.docx (cumulative) | v0.2.7 entry appended |
| briefs/WavefrontRefraction_Clinical_Spec_v2.md | Supersedes v1 — six-enhancement pass, doctor-led posture |
| xoExam Clinical Standards Reference v0.2.7.docx / .md | Reissued at v0.2.7 — Wavefront Refraction §7 updated for the six enhancements |
| xoExam Clinical Evaluation Brief.docx / .md | Reissued at v0.2.7 — Wavefront Refraction panel prompts updated |
| CLAUDE.md | v0.2.7 entry appended; WFR section rewritten (merge); component index + test-stream table updated |
| README.md | Status bumped to v0.2.7 |
| _dist_v0.2.7/ | Deployment package |

16. Version history

| Spec version | Date | Trigger |
| --- | --- | --- |
| v0.1.8 | May 18, 2026 | Visual Fields clinical fidelity + first formal handoff issue |
| v0.1.9 | May 21, 2026 | Wavefront Aberrometry clinical fidelity + Doctor sign-off pattern introduced |
| v0.2.1 | May 22, 2026 | Extraocular Motility clinical fidelity + naming convention sweep |
| v0.2.5 | May 22, 2026 | Pupillometry clinical fidelity |
| v0.2.6 | June 2, 2026 | Wavefront Refraction — new two-stage refraction test; Certify & close as Rx-release system event |
| v0.2.7 | June 8, 2026 | Wavefront Refraction — six competitive-parity enhancements merged into the single production file; v2 fork eliminated |

Older versioned specs are retained in the repository so spec evolution is traceable for MPR / regulatory audit.

*Method Marketing Agency · xoExam UI/UX Engineering Handoff Specification · v0.2.8 · June 9, 2026*