# xoExam™ — Development Brief

## v0.2.5 · May 22, 2026 · Pupillometry · Beta scope CLOSED

Method Marketing Agency for Xenon Ophthalmics Inc.

## Executive summary

v0.2.5 brings Pupillometry to clinical fidelity. This is the sixth and final test in the originally-defined beta scope — the beta scope is now closed. Six of the device's nineteen planned exams are at clinical fidelity in the prototype: Visual Acuity, Color Vision, Visual Fields, Wavefront Aberrometry, Extraocular Motility, and Pupillometry.

The rebuild replaced a 237-line static-only sketch with a ~1100-line clinical-fidelity component conforming to the Component Interface Contract — wrapped in ExamShell, three-phase flow, OD-first bilateral sequence, Patient Classification banner, four-pattern clinical flag system, Doctor sign-off. Net-new clinical content over the legacy implementation: dynamic pupillary light reflex (PLR) sub-test, NPi (Neurological Pupil index) composite per NeurOptics NPi-200 reference, swinging-flashlight RAPD assessment with six-grade clinical scale, age-banded reference range lookup driven by patient age input, and the light-vs-dark anisocoria differential rule.

The full handoff documentation set has been reissued at v0.2.5: Engineering Handoff Specification, Clinical Standards Reference, Clinical Evaluation Brief. Seven new open clinical questions have been added to the engineering kickoff agenda — see §H of the Pupillometry clinical spec.

## What changed

### PupillometryTest.jsx — rebuilt onto ExamShell with clinical fidelity

Architecture-level changes:

ExamShell wrapper. Three-phase flow (ready = eye-mode + sub-test config + patient age input · testing = sub-step state machine · report). Internal cancel modal removed — ExamShell owns canonical Cancel UX (back arrow hidden during testing, labeled red "Cancel test" button takes over with confirm modal).

Sub-test state machine inside testing. Three sub-steps (static → dynamic → swinging). Static and dynamic each iterate the OD-first bilateral eye sequence with a full-screen PUP_TransitionPrompt between eyes (patient-repositioning copy + eye-sequence breadcrumb). Swinging runs as a single screen with alternating-stimulus animation and per-eye RAPD graders.

OD-first bilateral sequence with PUP_InlineEyePicker / PUP_EyeBreadcrumb / PUP_TransitionPrompt — pattern adapted from WFA v0.1.9, VF v0.1.8, EOM v0.2.1.

All top-level identifiers prefixed PUP_ per the cross-file naming-collision rule.

Light-theme chrome matching the other clinical-fidelity tests; the iris viewer itself stays dark (clinically appropriate for the dim headset interior).

### Clinical-accuracy additions (5 major)

Age-banded reference range. Patient age input on the setup screen drives a Winn et al. 1994 lookup table — six age bands (20s through 70+) × three light conditions (scotopic / mesopic / photopic), mean ± 1 SD. Captured values flag (↯ icon, red text) when outside the age-band range, both in the testing sub-bar tiles and in the report's Static measurements table. Full Winn table surfaces in the report's Age-banded reference values card with the patient's band highlighted.

Anisocoria interpretation upgraded. Legacy code used >1mm = significant at a single light condition — missed the most important clinical signal (whether the anisocoria changes with light). v0.2.5 adds:

- Anisocoria computed at each of the three light conditions (color-coded per severity band)

- Dedicated light-vs-dark differential row in the Static measurements table — |aniso(dark) − aniso(light)|, flagged red at ≥ 0.3 mm with "≥ 0.3 mm = pathological pattern" annotation

- PUP_getAnisocoriaSeverity + PUP_getClinicalFlag use the differential to distinguish sympathetic (Horner's, worse-in-dark) from parasympathetic (CN III, Adie's, pharmacologic, worse-in-light) patterns

Dynamic pupillary light reflex sub-test added. Net-new clinical content. PUP_LightReflexChart renders an SVG line graph of pupil size vs. time over a 3-second window with stimulus marker, baseline reference line, peak-constriction annotation. Six summary metric tiles (baseline / min / constriction % / latency / constriction velocity / T75 redilation). Production firmware will emit the real time-series; the prototype computes a piecewise-modelled curve from PUP_simulateDynamics.

NPi (Neurological Pupil index) composite. Net-new clinical metric. PUP_computeNPi produces a 0–5 score from the dynamic measurement; PUP_npiSeverity thresholds per NeurOptics NPi-200 reference (≥ 3.0 normal / 2.5–3.0 borderline / < 2.5 abnormal). Surfaced per eye in the dynamic sub-test as a prominent severity-tinted tile and again in the report's Dynamic measurements card.

Swinging flashlight (RAPD) sub-test added. Net-new clinical content. PUP_SwingingScreen runs an alternating-stimulus animation across two iris viewers (3-second dwell, configurable via Start/Pause stimulus button). Each eye gets its own six-option grade selector — None / Trace / 1+ / 2+ / 3+ / 4+ — with descriptive copy at the bottom and per-eye severity bands.

### Report-level additions

Patient Classification banner with worst-finding-drives-bottom-line severity (normal / mild / significant) + per-finding summary pills (anisocoria · NPi per eye · RAPD).

Four-pattern clinical flag system in the report — each pattern emits its own labeled red-surface flag with specific differential and referral copy:

- Sympathetic Horner's pattern (anisocoria greater in dark)

- Parasympathetic CN III / Adie's pattern (anisocoria greater in light)

- Optic nerve RAPD pattern (RAPD ≥ 1+ on either eye)

- Neurological NPi pattern (NPi < 2.5 on either eye)

Clinical Interpretation card with severity tint + finding-by-finding bullet list (each age-banded deviation, each pathological anisocoria pattern, each NPi finding, each RAPD finding enumerated).

Age-banded reference values card with the full Winn table and the patient's age band highlighted — so the printed report stands alone.

### Visual sweep

Legacy linear-gradient(135deg,accent,#155bcc) Begin Test button replaced with solid accent (matches the no-gradient convention established in v0.1.8+).

Vivid purple / blue / amber light-level pills replaced with brand-palette segmented control. Severity coding doesn't apply to light conditions (scotopic isn't "purple severity"), so the old palette was clinically misleading.

Emoji-style ⚠ Significant / ✓ Normal indicators replaced with SVG icons throughout.

"Save Results" / "Print Report" action buttons replaced with the canonical "Export report" / "Compare" / "Certify & close" trio.

Doctor sign-off label above Certify & close, matching the v0.1.9 pattern established for clinical-fidelity tests.

Sentence-case sweep across all buttons and labels.

## Beta scope status

The originally-defined beta scope is now CLOSED. All six tests have been brought to clinical fidelity:

#  ·  Test  ·  Clinical-fidelity version

|---|---|---|

1  ·  Visual Acuity  ·  v0.2.3

2  ·  Color Vision (Ishihara + D-15 Farnsworth)  ·  v0.1.7

3  ·  Visual Fields  ·  v0.1.8

4  ·  Wavefront Aberrometry  ·  v0.1.9

5  ·  Extraocular Motility  ·  v0.2.1

6  ·  Pupillometry  ·  v0.2.5

The remaining 13 tests in the catalog are visual-fidelity placeholders awaiting their own clinical rebuilds in post-beta releases. The clinical evaluation panel's input on which of the 13 to prioritise next is welcomed.

## New open clinical questions (from Pupillometry §H)

These are added to the engineering kickoff agenda:

Dynamic data shape from firmware. Time-series at ~30 Hz, or only summary metrics?

NPi computation location. Firmware-side (NeurOptics-style) or UI/cloud-side from the time-series?

Swinging flashlight automation. Headset-driven alternation, or technician-driven?

Pupil response clicker. Pupillometry is passive — confirm clicker is not used.

Pharmacologic pupillometry workflow. Cocaine / apraclonidine / pilocarpine protocols — Xenon's roadmap?

Ambient light measurement. Real headset-interior lux readback, or assume calibrated dark?

Adaptation time enforcement. Firmware-enforced ≥ 5 min dark adaptation, or UI countdown timer needed?

## Document set issued at v0.2.5

Document  ·  Status

|---|---|

xoExam UI-UX Engineering Handoff Specification v0.2.5.docx  ·  New — supersedes v0.2.1

xoExam Development Brief 05-22-2026 v0.2.5.docx  ·  New release-specific brief

xoExam Development Brief.docx (cumulative)  ·  Updated with v0.2.5 entry

xoExam Clinical Standards Reference.md / .docx  ·  Updated to v0.2.5 — added Pupillometry §6, updated scope-of-fidelity table

xoExam Clinical Evaluation Brief.md / .docx  ·  Updated — added Pupillometry §6, removed from "not yet at clinical fidelity" list

briefs/Pupillometry_Clinical_Spec_v2.md  ·  Authoritative clinical spec (already in repo since pre-rebuild scoping)

CLAUDE.md  ·  v0.2.5 entry appended to version log; test stream status table updated; beta scope marked CLOSED

README.md  ·  Status bumped to v0.2.5

## Deployment

Code: components/PupillometryTest.jsx (replaced)

Title bumped: index.html → v0.2.5

Deploy harness: deploy.html → v0.2.5

Distribution package: _dist_v0.2.5/

Live: [xoexam-uiux.netlify.app](https://xoexam-uiux.netlify.app)

Method Marketing Agency · xoExam UI/UX Development Brief · v0.2.5 · May 22, 2026